# Parameter Exchange

Below documents describe the message structure of the parameter exchange implemented between the software and the PLC controller.

XX XXXX XX

Resource address

Sub resource address

Offset address

XX XXXX XX

Resource address

Sub resource address

Offset address

Every value is paired with a key (identification number).
This ensures that every machine uses the same key for the same value, irrespective of the project it belongs to.

Key Structure (32-bit)

The 32-bit key is arranged as follows:

| Bits | Meaning |
| --- | --- |
| [31:24] (8 bits, MSB) | Resource Number |
| [23:16] (8 bits) | Sub-resource Number |
| [15:0] (16 bits) | Offset Address |

Resource Examples

Resources represent major components of the machine, such as:

- Axis

- Robot

- Stations (inputs/outputs)

- Gripper

- Cylinder

- Valves

- etc.

Within each resource:

- The resource number identifies the type of component

- The sub-resource number identifies the instance/element within that component

- The offset address represents the specific parameter or value within that instance

Example

If:

- The resource ID of the axis is 100,

- The sub-resource index is 4 (e.g., 4th axis),

- The offset address for current position is 3,

Then the key for accessing the current position of the 4th axis is:

key = (100 << 24) | (4 << 16) | 3

This packs the resource number, sub-resource number, and offset into a single 32-bit identifier.

In our system, when the PC software starts, it first queries the PLC for the parameterKeys associated with that specific machine.
This initial exchange provides a complete list of all parameter keys that are relevant to the machine configuration.

Startup Phase

- PC → PLC: Request the list of parameterKeys.

- PLC → PC: Transmit the ordered list of parameter keys for that machine.

This ensures that:

- The PC knows exactly which parameters are available.

- The PC knows the sequence/order in which parameter values will be transmitted.

- Different machines or configurations remain compatible, because the keys—not fixed positions—define meaning.

Cyclic Read Phase

After the initial parameterKeys list is received, the PC begins cyclic reading of parameter values from the PLC.

During cyclic reading:

- The PLC sends parameter values in the same order as the parameterKeys list delivered during startup.

- The PC maps each value back to its meaning using the previously received key list.

This approach ensures:

- Flexibility (machines can have different configurations).

- Stability (same key always means the same parameter).

- Efficient communication (only values are exchanged cyclically; structure is exchanged once).

Table: Resource Key Definitions

| Resource | Value | Description |
| --- | --- | --- |
| Version | 1 | Version information for the parameter set |
| Machine | 10 | Machine-level configuration and status parameters |
| Digital Inputs | 11 | Digital input status as a pack of 32 |
| Digital Outputs | 12 | Digital output status as a pack of 32 |
| Analog Inputs | 13 | Analog inputs |
| Analog outputs | 14 | Analog outputs |
| Axis | 100 | Axis-related parameters (position, status, commands) |
| Robot | 120 | Robot-related parameters |
| Station | 121 | Station parameters |
| Gripper | 122 | Gripper-related parameters |
| Cylinder | 123 | Cylinder actuator parameters |
| Emergency | 150 | Emergency/safety-related parameters |
| Tank | 151 | Tank-level parameters (levels, sensors, etc.) |
| Pressure | 152 | Pressure sensor or pressure system parameters |
| Valve | 153 | Valve control and status parameters |
| Flow | 154 | Flow sensor parameters |
| LiftStation | 200 | Lift station–related parameters |
| CenterStation | 201 | Centering station parameters |
| Conveyor | 202 | Conveyor parameters |
| Laser Mark Station | 203 | Laser marker parameters |
| Notification | 253 | Notification from the device |
| Project | 254 | Project-level metadata and configuration |

1. Machine – Offset Address Table

| Offset Name | Value | Description |
| --- | --- | --- |
| AccessLevel | 1 | Machine access level |
| NoOfRealAxis | 2 | Number of real axes configured |
| NoOfVirtualAxis | 3 | Number of virtual axes configured |
| NoOfDI | 4 | Number of digital inputs |
| NoOfDO | 5 | Number of digital outputs |
| CamMasterAxisIndex | 6 | Master axis index for camming |
| NoOfEmergency | 7 | Number of emergency devices |
| AxisStartAddress | 8 | Start address for axis parameters |
| EmergencyStartAddress | 9 | Start address for emergency parameters |
| NoOfWaterTanks | 10 | Number of water tanks |
| WaterTankStartAddress | 11 | Start address for water tank parameters |
| NoOfValves | 12 | Number of valves |
| ValveStartAddress | 13 | Start address for valve parameters |
| NoofNotifications | 14 | No of current notifications in the machine |
| NewNotificationActive | 15 | New notification present after last read |
| plcToPcWatchdog | 16 | Variable holds plc to pc heartbeat |
| AutomationCycle running | 17 | Indicates whether the automation sequence cycle running in PLC/Controller or not |

PcToPLCWathdog is present in PLC to echo the signal received in plcToPcWatchdog variable.

2. Gripper – Offset Address Table

| Offset Name | Value | Description |
| --- | --- | --- |
| Type | 1 | Type of gripper |
| Status | 2 | Current status |
| ArmNumber | 3 | Number of arms |
| Arm1AxisIndex | 4 | Axis index of arm 1 |
| Arm2AxisIndex | 5 | Axis index of arm 2 |
| IsGripperReleased | 6 | Release status |
| IsGripperGripped | 7 | Grip status |
| IsSamplePresent | 8 | Workpiece/sample presence |
| Error | 9 | Error flag |
| Arm1Pos | 10 | Position of arm 1 |
| Arm2Pos | 11 | Position of arm 2 |
| ErrorCode | 12 | Error code |

3. Centering Station – Offset Address Table

| Offset Name | Value | Description |
| --- | --- | --- |
| Type | 1 | Station type |
| Status | 2 | Current status |
| ArmNumber | 3 | Number of arms |
| Arm1AxisIndex | 4 | Axis index of arm 1 |
| Arm2AxisIndex | 5 | Axis index of arm 2 |
| EngagedStatus | 6 | Indicates engaged state |
| DisengagedStatus | 7 | Indicates disengaged state |
| IsSamplePresent | 8 | Sample presence |
| Error | 9 | Error flag |
| Arm1Pos | 10 | Position of arm 1 |
| Arm2Pos | 11 | Position of arm 2 |
| ErrorCode | 12 | Error code |
| CenterPosition | 13 | Centering position value |

4. Lift Station – Offset Address Table

| Offset Name | Value | Description |
| --- | --- | --- |
| Type | 1 | Station type |
| Status | 2 | Current status |
| ArmNumber | 3 | Number of arms |
| Arm1AxisIndex | 4 | Axis index of arm 1 |
| DownPosition | 5 | Down position value |
| UpPosition | 6 | Up position value |
| DisengagedStatus | 7 | Disengaged flag |
| IsSamplePresent | 8 | Sample presence |
| Error | 9 | Error flag |
| Arm1Pos | 10 | Position of arm 1 |
| Arm2Pos | 11 | Position of arm 2 |
| ErrorCode | 12 | Error code |
| CenterPosition | 13 | Centering reference |

5. IMT / Input / Output Station

| Offset Name | Value | Description |
| --- | --- | --- |
| Type | 1 | Station type |
| Component loaded to | 2 | Component loaded |
| ComponentPresentSensor state | 3 | Component presence detected by sensor |
| Component Unloaded from | 4 | Component unloaded |

6. Emergency – Offset Address Table

| Offset Name | Value | Description |
| --- | --- | --- |
| Location | 1 | Emergency device location |
| Active | 2 | Whether emergency is active |
| Error | 3 | Error status |

7. Water Tank – Offset Address Table

| Offset Name | Value | Description |
| --- | --- | --- |
| IdentIndex | 1 | Tank identification number |
| LevelOk | 2 | Water level OK status |
| LevelInM | 3 | Current water level (m) |
| MinLevelInM | 4 | Minimum threshold (m) |
| MaxLevelInM | 5 | Maximum threshold (m) |

8. Pressure – Offset Address Table

| Offset Name | Value | Description |
| --- | --- | --- |
| LevelOk | 1 | Pressure OK status |
| LevelInBar | 2 | Pressure in bar |
| MaxThreshold | 3 | Max pressure threshold |
| MinThreshold | 4 | Min pressure threshold |

9. Valve – Offset Address Table

| Offset Name | Value | Description |
| --- | --- | --- |
| Location | 1 | Valve location |
| Closed | 2 | Closed state |
| OpenPercent | 3 | Valve opening in % |
| Error | 4 | Error state |

8. Flow– Offset Address Table

| Offset Name | Value | Description |
| --- | --- | --- |
| Flow ok | 1 | Flow OK status |
| Flow in l/m | 2 | Flow in l/m |
| MaxThreshold | 3 | Max flow threshold |
| MinThreshold | 4 | Min flowthreshold |

10. Cylinder – Offset Address Table

| Offset Name | Value | Description |
| --- | --- | --- |
| IsInExpandCondition | 1 | Cylinder fully extended |
| IsInRetractCondition | 2 | Cylinder fully retracted |
| OperationStatus | 3 | Operation status |
| CylinderPosition | 4 | Cylinder position in mm |
| Cylinder state | 5 | Cylinder state |

11. Axis – Offset Address Table

| Offset Name | Value | Description |
| --- | --- | --- |
| AxisName | 1 | Axis Name |
| AxisIndex | 2 | Axis index |
| CurrPosition | 3 | Current position |
| CurrStatus | 4 | Axis status |
| CurrTorque | 5 | Current torque |
| CurrVelocity | 6 | Current velocity |
| CurrentInAmp | 7 | Motor current (A) |
| CurrAcceleration | 8 | Current acceleration |
| CurrDeceleration | 9 | Current deceleration |
| SoftLimitEnabled | 10 | Soft limit enabled flag |
| FwdSoftLimit | 11 | Forward soft limit |
| RevSoftLimit | 12 | Reverse soft limit |
| IsDualAxis | 13 | Whether dual-axis |
| NoOfDualAxis | 14 | Count of dual axes |
| DualAxis1 | 15 | Dual axis 1 |
| DualAxis2 | 16 | Dual axis 2 |
| DualAxis3 | 17 | Dual axis 3 |
| DualAxis4 | 18 | Dual axis 4 |
| DualAxis5 | 19 | Dual axis 5 |
| MinVelocity | 20 | Minimum velocity |
| MaxVelocity | 21 | Maximum velocity |
| MinAcc | 22 | Minimum acceleration |
| MaxAcc | 23 | Maximum acceleration |
| MinDec | 24 | Minimum deceleration |
| MaxDec | 25 | Maximum deceleration |
| JogHighVelocity | 26 | Jog high speed |
| JogMedVelocity | 27 | Jog medium speed |
| JogLowVelocity | 28 | Jog low speed |
| JogHighAcc | 29 | Jog high acceleration |
| JogMedAcc | 30 | Jog medium acceleration |
| JogLowAcc | 31 | Jog low acceleration |
| JogHighDec | 32 | Jog high deceleration |
| JogMedDec | 33 | Jog medium deceleration |
| JogLowDec | 34 | Jog low deceleration |
| NoOfErrorsPresent | 40 | Number of errors |
| ActiveError1 | 41 | Error 1 |
| ActiveError2 | 42 | Error 2 |
| ActiveError3 | 43 | Error 3 |
| ActiveError4 | 44 | Error 4 |
| ActiveError5 | 45 | Error 5 |
| ActualCurrent | 110 | Motor current |
| ActualTorque | 111 | Motor torque |
| ActualTemp | 112 | Motor temperature |
| ActualSpeedRPM | 113 | Motor speed in RPM |

10. Robot – Offset Address Table

| Offset Name | Value | Description |
| --- | --- | --- |
| Robot model | 1 |  |
| IsAtHomePosition | 2 | Robot at home position |
| CurrentRobotPosPoint | 3 | Depends on project where is robot current position |
| Robot Operation state | 4 | Operation status |
| Robot Current state | 5 | Idle/Interrupted |
| Component Id selected on robot | 6 | Component id wrt that project. |
| Robot mode | 7 | Manual/Auto |
| Robot Joint1 pos | 10 |  |
| Robot Joint2 pos | 11 |  |
| Robot Joint3 pos | 12 |  |
| Robot Joint4 pos | 13 |  |
| Robot Joint5 pos | 14 |  |
| Robot Joint6 pos | 15 |  |

10. Notification – Offset Address Table (sub resource 1)

| Offset Name | Value | Description |
| --- | --- | --- |
| Notification id/Resource | 1 |  |
| Identification number | 2 |  |
| Notification severity | 3 |  |
| Timestamp | 4 |  |
| Message |  |  |

5. Marker

| Offset Name | Value | Description |
| --- | --- | --- |
| Type | 1 | Marker type – Laser/Dotpeen |
| Marking status | 2 | Status of operation |
| Error | 3 | Error status |

# 2Parameter Key & Value Exchange Mechanism3

## Parameter read procedure in Twincat

The PC reads parameter data from the PLC using a predefined array-based communication structure.
This ensures consistent, ordered, and scalable data exchange across different machine configurations.

1. Reading Parameter Keys (Initialization Phase)

When communication starts, the PC first reads the ParameterKey array from the PLC.

- Index 0 of the array contains the total number of parameter keys exchanged.

- The next N entries (where N is the count at index 0) contain the actual parameter keys, arranged in the order defined by the PLC.

Structure Example:

| Array Index | Meaning |
| --- | --- |
| 0 | Number of keys (N) |
| 1 | Key 1 |
| 2 | Key 2 |
| 3 | Key 3 |
| … | … |
| N | Key N |

This initial read allows the PC to understand:

- Which parameters are available

- The unique keys mapped to each parameter

- The exact order in which values will be transmitted cyclically

2. Cyclic Reading of Parameter Values

After the initial key list is received, the PC begins reading the ParameterValue array cyclically.

- The values in this array are always arranged in the same order as the keys received during initialization.

- Therefore, the PC can reliably map each value to its respective variable using the previously stored parameter key list.

Mapping Logic:

Value[i] →  ParameterKey[i]

Each value is assigned to the correct internal variable by:

- Looking up the key at index i

- Mapping it to the corresponding resource, subresource, and offset

- Updating the related software variable

## Parameter read procedure in a TCP device

Similarly, in a TCP command–based system, the 506 query command is transmitted to the PLC over the TCP socket connection. The PLC processes the incoming command and sends back the response using the same structure and mechanism as described in the TwinCAT section. The only difference is the communication medium (TCP) instead of ADS/TwinCAT channels.

The 508 query command is used to request parameter key–value information.
Upon receiving this command, the PLC sends back all available parameter keys with their mapped values, allowing the client system to dynamically read configuration parameters, limits, or operational settings.

Drive parameter address for various drives

| Parameter | Schneider | ZeroErr | Festo | Beckhoff | Synapticon |
| --- | --- | --- | --- | --- | --- |
| Actual Torque | 6077:0h (0.1 increments, 100% - Stall torque) |  |  |  |  |
| Motor temperature |  |  |  |  |  |
| Total RMS current | 301E:3h (0.01 Arms) |  |  |  |  |
| Motor power | 301C:Eh |  |  |  |  |
| Motor rotation in rpm | 301E:8h |  |  |  |  |